IOPGPS Config Tool 3.0 User Manual

Applicable Version: Config Tool 3.0

Purpose: Software installation, device connection, parameter configuration, and command sending

1. Product Overview

IOPGPS Config Tool 3.0 is a configuration tool for WanWay GPS tracker devices. It is mainly used for device detection, parameter configuration, and command sending.

After a tracker is connected, the tool automatically detects it and maintains the connection. Use Template Commands to configure common parameters through the visual interface, or Custom Commands to send command text directly to the tracker.

1.1 Supported Hardware

- GS900

- GS10G

- GS950

- ENTRACK VL100

- GS925

- GS12

Supported template parameters and commands may vary by tracker model. Refer to the options displayed for the connected tracker and the corresponding device protocol.

2. Software Installation and Startup

2.1 Before You Begin

Before using the tool, make sure that:

- You have received the complete Config Tool package and driver package.

- Extract the entire Config Tool package to a local folder before running it. Do not run the program directly from the compressed archive.

- Connect the tracker to the computer.

- If the computer does not recognize the tracker, install the provided driver and reconnect the tracker.

2.2 Launch the Software

- Open the extracted Config Tool folder.

- Locate IopConfig.Desktop.exe.

- Double-click the file to launch IOP Config Tool.

3. Interface Overview

Use the top navigation to switch between Template Commands, Custom Commands, and Devices. Use the top device bar to scan and switch trackers.

| Area / Page | Purpose |
| --- | --- |
| Top Device Bar | Scan and switch connected trackers. |
| Template Commands | Configure parameters and send template commands. |
| Custom Commands | Manage Command Sets and send custom commands. |
| Devices | View connected trackers and device details. |

4. Device Management

4.1 Automatic Device Detection

When a tracker is connected to the computer, the tool automatically scans for and identifies it. You do not need to manually open or close the connection.

Procedure:

- Connect the tracker to the computer.

- Launch IOP Config Tool.

- Wait for the tool to detect the tracker automatically.

- Confirm that the tracker appears on the Devices page or in the IMEI selector on the top device bar.

- Select the target tracker, then open Template Commands or Custom Commands to configure it.

Newly connected trackers are detected automatically. Repeated manual refresh is not normally required.

4.2 Manual Scan

If a connected tracker does not appear, click Scan Devices on the top device bar to scan all currently connected trackers again.

4.3 Switch Devices

To work with another tracker, use the IMEI drop-down on the top device bar to switch the active device. You do not need to open the Devices page first.

5. Template Commands

5.1 Overview

Template Commands is used to configure common tracker parameters. Select or enter values in the visual parameter cards; the tool automatically generates the corresponding device commands, so you do not need to edit command text manually.

Supports:

- Sending commands to one device

- Sending commands to multiple devices

- Sending a single parameter command

- Sending all template commands

5.2 Configure a Single Device

Use case: Change one or more parameters on a single tracker.

Procedure:

- Select the target IMEI from the top device bar.

- Open Template Commands.

- Set the required parameters.

- Verify the target tracker and parameter values.

- Click Send on an individual parameter card, or click Send to Device to send all template commands to the selected tracker.

- Check Communication Log and the result window to confirm the configuration.

5.3 Send a Single Parameter Command

Use case: Change only one parameter without resending other settings.

Procedure:

- Locate the parameter card you want to change.

- Set the required value.

- Click Send on that parameter card.

- Review Communication Log and the device response to confirm the result.

5.4 Send All Template Commands to One Device

Use case: Send the complete set of template commands to the currently selected tracker.

Procedure:

- Complete the required parameter settings.

- Verify the parameter values and the target tracker.

- Click Send to Device.

- In the Send All Template Commands window, monitor progress and review the command results.

5.5 Send Template Commands to Multiple Devices

Use case: Configure the same parameters on multiple compatible trackers to reduce repeated work.

Procedure:

- Open Template Commands.

- Make sure all target trackers are connected and detected.

- Set the parameters to be sent.

- Verify the target device range and parameter values.

- Click Send to All Devices.

- In the Send All Template Commands window, review All Devices List and This Run to confirm per-device progress and command results.

Note: Before sending to all devices, carefully verify the target devices and configuration values to avoid applying the same parameters to devices that should not be changed.

6. Custom Commands

6.1 Overview

Custom Commands is intended for cases where you already know the exact command text and need to send it directly to trackers. You can enter command text without using template parameters.

The page supports multiple command sets. Commands for different devices or use cases can be saved separately and switched by selecting the corresponding Command Set, without importing them every time.

6.2 Send Custom Commands

Procedure:

- Open Custom Commands.

- Select the required Command Set, or create a new one.

- Enter or paste the command text in the command editor.

- In Device List, select the target device or devices and verify the target IMEIs.

- Click Send to Device.

- Review Progress, Result, and Send History to confirm the execution result.

6.3 Add and Switch Command Sets

You can maintain multiple command sets when different groups of commands are required.

Procedure:

- Open Custom Commands.

- Click the + icon next to COMMAND SET to add a command set, or click Import to import one.

- Add or edit the command text for the selected Command Set.

- To switch later, click the required Command Set in the list on the left.

Note: Custom Commands sends the entered text directly to the selected tracker(s). Before sending, verify the command format, parameter values, and tracker model.

7. Recommended Workflow

For routine tracker configuration, follow this sequence:

- Connect the tracker(s) to the computer.

- Launch IOP Config Tool and confirm that the tracker(s) are detected automatically.

- Select a single tracker from the top device bar, or select multiple target devices in the relevant page.

- Use Template Commands for common parameter configuration.

- Use Custom Commands when you need to send special commands or known command text.

- Before sending, verify the target device(s), parameter values, and command content.

- After sending, review Communication Log, Result, or Send History to confirm whether the operation succeeded.

8. FAQ

8.1 The tracker is connected to the computer, but IOP Config Tool does not detect it

Check the following in order:

- Make sure the physical connection between the tracker and the computer is secure.

- Make sure the required driver is installed correctly.

- Reconnect the tracker and wait for the tool to detect it automatically.

- Click Scan Devices on the top device bar to scan connected trackers again.

8.2 How can I quickly apply the same parameters to multiple trackers?

Use Send to All Devices on Template Commands. Verify the connected target trackers and parameters before sending.

8.3 Do I need to send all template commands if I only want to change one parameter?

No. Click Send on the corresponding parameter card to send only the parameter you want to change.

8.4 Do I need to re-import multiple custom command sets every time?

No. IOP Config Tool supports multiple Command Sets. Select the required Command Set from the list on the left.

8.5 Do I need to manually open or close the device connection?

No. The current version automatically detects connected trackers and maintains the device connection.